EMBEDDED LAB REPORT

Table of Contents

| S.No | Topic | Lab | Date | Page No |
| --- | --- | --- | --- | --- |
| 1 | Blink LEDs in alternate order | 1 | 31/01/2022 |  |
| 2 | Blink LEDs – Alternate Wait |  |  |  |
| 3 | Blink LEDs – Composite Numbers |  |  |  |
| 4 | Blink LEDS – Even Numbers |  |  |  |
| 5 | Hex and Decade Counters – Switch | 2 | 07/02/2022 |  |
| 6 | Octal Counter – Ticker and Timeout |  |  |  |
| 7 | Odd and Even Counter – Ticker, Timeout and Switch |  |  |  |
| 8 | Intensity of LED - Pwmout | 3 | 14/02/2022 |  |
| 9 | Intensity of LED - Potentiometer |  |  |  |
| 10 | Intensity/Blinking of LEDs – Potentiometer, Switch |  |  |  |
| 11 | LED and Speaker – Pwmout | 4 | 28/02/2022 |  |
| 12 | Alarm System – Double Beep every 5s |  |  |  |
| 13 | LEDs and Speaker – Increase and Decrease intensity/frequency using switch |  |  |  |
| 14 | Simulate Ambulance sound |  |  |  |

| S.No | Topic | Lab | Date | Page No |
| --- | --- | --- | --- | --- |
| 15 | Tinkercad – Blink LEDs in alternate order | 5 | 07/03/2022 |  |
| 16 | Tinkercad – Blink LEDs – Alternate Wait |  |  |  |
| 17 | Tinkercad – Blink LEDs – Composite |  |  |  |
| 18 | Tinkercad – Blink LEDs – Even Numbers |  |  |  |
| 19 | Ultrasonic Sensor | 6 | 28/03/2022 |  |
| 20 | PIR Sensor |  |  |  |
| 21 | 7-Segment Display Odd and Even Counter | 7 | 11/04/2022 |  |
| 22 | Ambient Light Sensor |  |  |  |
| 23 | Temperature and Gas Sensor |  |  |  |
| 24 | Force Sensor | 8 | 18/04/2022 |  |
| 25 | Keypad and Tilt Sensor |  |  |  |

Questions

MBED

1) Blink LEDs in alternate order – 1 and 3 together and 2 and 4 together.

2) Blink LEDs – count from 1 to 15; if the board is counting odd value, wait for 1 sec, else wait for 2 sec.

3) Blink LEDs – for all composite number below 15.

4) Blink LEDs – to count even numbers

5) Switch between hex counter and decade counter using switches. Display the current value of the counter with the help of 4 LEDs and the screen.

6) Display the octal counter using the onboard LEDs using Ticker Object; After 10 second, reset all the values of the onboard LED using Timeout Object.

7) Display the odd counter using the onboard LEDs and screen; After 10 seconds, reset all the values of the onboard LED and display even counter using Timeout Object. Give switch option to manually toggle between two counters.

8) Increase the intensity of one LED while decreasing the other LED's intensity at the same unit. Display the intensity value of the LEDs.

9) Interface potentiometer with mbed board and based on the value of potentiometer, adjust the intensity of an external LED.

10) Interface switch, 2 LEDs, potentiometer with the mbed board. If the switch is on, control the intensity of the LEDs using potentiometer (one LED should be increasing and other should be decreasing at the rate given via potentiometer). If the switch is off, blink LEDs one at increasing rate and other at decreasing rate (same rate) and loop back.

11) Write a program to increase the intensity of external LED. At the same rate, increase the sound intensity of speaker.Libraries Used: pwmout

12) Design an alarm system, which makes double beep sound for every 5s using a speaker. Libraries To Be Used: pwmout, analogin

13) Have a switch, 2 LEDs and a speaker interfaced with mbed board. Upon switching on, the intensity of the sound should be increasing and the upon switching off, should make the intensity of the sound decreasing. Proportionally increase or decrease the intensity of two LEDs.

14) Simulate ambulance sound using a speaker.

Tinkercad

15) Blink LEDs in alternate order – 1 and 3 together and 2 and 4 together.

16) Blink LEDs – count from 1 to 15; if the board is counting odd value, wait for 1 sec, else wait for 2 sec.

17) Blink LEDs – for all composite number below 15.

18) Blink LEDs – to count even numbers.

19) Interface an ultrasonic sensor and display the distance measured in a screen and glow red, blue and green led if the distance is close, moderate and far respectively. Play different tones for different cases.

20) Interface a PIR sensor with Arduino and blink a led if any movement is captured by the sensor and count the number of movements. Make a buzzer when movement detected.

21) Interface two 7-segment display with Arduino Uno board. Implement the a)odd counter and b)even counter and display the values in the seven-segment display.

22) Interface the ambient light sensor with Arduino Uno board. Check the light value from the sensor and switch on/off the bulb (based on the threshold value).

23) Interface the temperature and gas sensor with Arduino Uno board. Check the temperature and the gas value, if the limit is beyond the threshold, switch on the bulb and make alarm using buzzer.

24) Interface force sensor with Arduino board and display the amount of force given as input to sensor in LCD screen. Also, interface LED RGB and change color of the LED based on some threshold values.

25) Interface the keypad and tilt sensor with Arduino board, if the sensor is being tilted, then take input from keypad and print it in the LCD.

Experiment 1

Aim

To blink LEDs in alternate order – 1 and 3 should blink together and 2 and 4 should blink together.

Code

#include "mbed.h"

DigitalOut led1(LED1);

DigitalOut led2(LED2);

DigitalOut led3(LED3);

DigitalOut led4(LED4);

int main() {

led1 = led3 = 1;

led2 = led4 = 0;

while (1) {

led1 = !led1;

led3 = led1;

printf("Blink! LED is now %d\n", led1.read());

led2 = !led2;

led4 = led2;

printf("Blink! LED is now %d\n", led2.read());

wait_ms(500);

}

}

Output

Serial Output

Experiment 2

Aim

To blink LEDs and count from 1 to 15. If the board is counting odd value, it should wait for 1 sec, else it should wait for 2 sec.

Code

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

int main() {

leds = 0;

for(int i=1; i<=15; i++) {

leds = i;

if(i % 2 == 1) {

wait_ms(1000);

} else {

wait_ms(2000);

}

}

}

Serial Output

Experiment 3

Aim

To blink LEDs for all composite numbers below 15.

Code

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

bool isPrime(int k) {

if(k <= 1) {

return false;

}

for(int i=2; i<=sqrt(k); i++) {

if(k%i == 0)

return false;

}

return true;

}

int main() {

leds = 0;

for(int i=1; i<=15; i++) {

if(!isPrime(i))

leds=i;

printf("%d \n", i);

wait_ms(1000);

}

}

Output

Serial Output

Experiment 4

Aim

To blink LEDs to count even numbers

Code

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

int main() {

leds = 0;

for(int i=1; i<=15; i++) {

if(i%2 == 0) {

leds=i;

printf("Printing for %d\n", i);

}

wait_ms(1000);

}

}

Output

Serial Output

Experiment 5

Aim

To switch between hex counter and decade counter using switches and display the current value of the counter with the help of 4 LEDs and the screen using the InterruptIn and BusOut libraries.

Code

#include "mbed.h"

DigitalOut led(LED1);

InterruptIn btn(p5);

BusOut leds(LED1, LED2, LED3, LED4);

void hex() {

while(1) {

for(int i=0; i<16; i++) {

leds = i;

printf("Hex: %d \n", i);

wait(1);

}

}

}

void dec() {

while(1) {

for(int i=0; i<10; i++) {

leds = i;

printf("Dec: %d \n", i);

wait(1);

}

}

}

int main() {

btn.rise(&hex);

btn.fall(&dec);

}

Output

Dec:

Hex:

Serial Output:

Dec:                                                                Hex:

2.

Experiment 6

• Display the octal counter using the onboard LEDs using Ticker Object; After 10 second, reset all the values of the onboard LED using Timeout Object.

• Libraries Used: TimeOut, Ticker

Aim

To display the octal counter using the onboard LEDs using Ticker Object and after 10 seconds reset all the values of the onboard LED using Timeout Object.

Code

#include "mbed.h"

BusOut leds(LED1, LED2, LED3);

Ticker tk;

Timeout to;

int current = 0;

void octal() {

leds = (current++)%8;

printf("Octal: %d \n", leds.read());

}

void reset() {

printf("Reset Done \n");

current=0;

leds=0;

}

int main() {

tk.attach(&octal, 1);

to.attach(&reset, 10);

}

Output

Serial Output

Experiment 7

Aim

To display the odd counter using the onboard LEDs and screen and after 10 seconds, reset all the values of the onboard LED and display even counter using Timeout Object with a switch option to manually toggle between two counters.

Code

#include "mbed.h"

InterruptIn my_switch(p5);

Timeout my_timeout;

Ticker my_ticker;

BusOut myleds(LED1, LED2, LED3, LED4);

int i = 0;

bool is_odd = 0;

void switch_odd() {

printf("switching to odd\n");

i = 1;

is_odd = true;

}

void switch_even() {

printf("switching to even\n");

i = 0;

is_odd = false;

}

void increase_count() {

myleds = i % 16;

i += 2;

printf("count: %d\n", myleds.read());

}

void reset() {

printf("resetting count\n");

switch_even();

}

int main() {

my_ticker.attach(&increase_count, 0.5);

my_timeout.attach(&reset, 10.0);

my_switch.rise(&switch_odd);

my_switch.fall(&switch_even);

}

Output

Serial Output

Experiment 8

Aim

To Increase the intensity of one LED while descreasing the other LED's intensity at the same unit and display the intensity value of the LEDs using pwmout library.

Code

#include "mbed.h"

PwmOut led1(p5);

PwmOut led2(p6);

int main() {

led1 = 0;

led2 = 1;

int flag = 1;

while (1) {

if(flag == 1) {

led1 = led1 + 0.10;

led2 = led2 - 0.10;

} else {

led1 = led1 - 0.10;

led2 = led2 + 0.10;

}

printf("Blink! LED1 is now %0.2f \n", led1.read());

printf("Blink! LED2 is now %0.2f \n", led2.read());

wait(0.1);

if(led1 == 1.0) {

flag = 0;

}

if(led1 == 0.0) {

flag = 1;

}

}

}

Output

Serial Output

Experiment 9

Aim

To Interface potentiometer with mbed board and based on the value of potentiometer, adjust the intensity of an external LED using pwmOut and analogIn libraries.

Code

#include "mbed.h"

AnalogIn p(p15);

PwmOut led(p5);

int main() {

float var;

while (1) {

var = p.read();

led = var;

printf("LED is now %0.2f \n", led.read());

wait(1);

}

}

Output

Serial Output

Experiment 10

Aim

To Interface switch, 2 LEDs, potentiometer with the mbed board and If the switch is on, control the intensity of the LEDs using potentiometer (with one LED increasing and the other decreasing at the rate given via potentiometer) and if the switch is off, blink LEDs one at increasing rate and other at decreasing rate (same rate) and loop back.

Code

#include "mbed.h"

AnalogIn p(p15);

PwmOut led1(p5);

PwmOut led2(p6);

InterruptIn btn(p7);

void on() {

float var;

while(1) {

var = p.read();

led1 = var;

led2 = 1-var;

printf("LED1 is %0.2f. LED2 is %0.2f \n", led1.read(), led2.read());

wait(0.2);

}

}

void off() {

led1 = 0;

led2 = 1;

int flag = 1;

while (1) {

if(flag == 1) {

led1 = led1 + 0.10;

led2 = led2 - 0.10;

} else {

led1 = led1 - 0.10;

led2 = led2 + 0.10;

}

printf("Blink! LED1 is now %0.2f \n", led1.read());

printf("Blink! LED2 is now %0.2f \n", led2.read());

wait(0.1);

if(led1 == 1.0) {

flag = 0;

}

if(led1 == 0.0) {

flag = 1;

}

}

}

int main() {

btn.rise(&on);

btn.fall(&off);

}

Output

Serial Output

Experiment 11

Aim

To increase the intensity of external LED and at the same rate, increase the sound intensity of speaker using pwmOut library.

Code

#include "mbed.h"

PwmOut led(p5);

PwmOut speaker(p21);

void play_tone(float frequency, float volume, int interval, int rest) {

speaker.period(1.0 / frequency);

speaker = volume;

wait(interval);

speaker = 0.0;

wait(rest);

}

int main()

{

led=0.0;

float vol=0.0;

while(1) {

led = led + 0.1;

vol += 0.1;

printf("led=%f\n", led.read());

printf("vol=%f\n", vol);

play_tone(200.0, vol, 1, 0);

wait(1);

if(led==1.0){

led=0.0;

vol=0.0;

}

}

}

Output

Serial Output

Experiment 12

Aim

To design an alarm system, which makes double beep sound for every 5s using a speaker using pwmout and analogin libraries.

Code

#include "mbed.h"

Ticker tk;

PwmOut speaker(p21);

void beep(){

speaker=0.3;

wait(0.1);

speaker=0;

wait(0.1);

speaker=0.3;

wait(0.1);

speaker=0;

}

int main()

{

tk.attach(&beep,5.0);

}

Output

Serial Output

Experiment 13

Aim

To have a switch, 2 LEDs and a speaker interfaced with mbed board and upon switching on, increase the intensity of the sound and the upon switching off decrease the intensity of the sound and proportionally increase or decrease the intensity of two LEDs.

Code

#include "mbed.h"

PwmOut speaker(p21);

PwmOut led1(p5);

PwmOut led2(p6);

InterruptIn btn(p7);

float sign = -0.1;

void inc(){

sign = 0.1;

}

void dec(){

sign = -0.1;

}

int main()

{

btn.rise(&inc);

btn.fall(&dec);

speaker=0;

while(1) {

if(sign>0 && speaker>=1)

speaker=0;

if(sign<0 && speaker<=0)

speaker=1;

led1 = speaker;

led2 = speaker;

speaker = speaker + sign;

wait(0.2);

}

}

Output

Serial Output

Case 1

Case 2

Experiment 14

Aim

To simulate the ambulance sound using the PwmOut library.

Code

#include "mbed.h"

PwmOut speaker(p21);

int main(){

int i;

while(1){

for(i=0; i <26; i++){

speaker.period(1.0/259.0);

speaker = float(i)/500.0;

wait(0.5);

speaker.period(1.0/340.0);

wait(0.5);

}

for(i=26; i >= 0; i--){

speaker.period(1.0/259.0);

speaker = float(i)/500.0;

wait(0.5);

speaker.period(1.0/440.0);

wait(0.5);

}

}

}

Output

Experiment 15

Aim

To blink LEDs in alternate order – 1 and 3 should blink together and 2 and 4 should blink together.

Tinkercad Link

https://www.tinkercad.com/things/1PwYTabHSMP

Code

// C++ code

//

void setup()

{

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

digitalWrite(8, HIGH);

digitalWrite(10, HIGH);

digitalWrite(9, LOW);

digitalWrite(11, LOW);

delay(1000); // Wait for 1000 millisecond(s)

digitalWrite(8, LOW);

digitalWrite(10, LOW);

digitalWrite(9, HIGH);

digitalWrite(11, HIGH);

delay(1000); // Wait for 1000 millisecond(s)

}

Output

Experiment 16

Aim

To blink LEDs and count from 1 to 15. If the board is counting odd value, it should wait for 1 sec, else it should wait for 2 secs.

Tinkercad Link

https://www.tinkercad.com/things/fDj0RiOP8Hf

Code

// C++ code

//

void setup()

{

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void display(int n)

{

digitalWrite(8, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(9, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(10, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(11, n&1?HIGH:LOW);

}

void loop(){

for(int i=0;i<16;i++)

{

display(i);

Serial.print("printing ");

Serial.print(i);

Serial.print(" with a delay of ");

if(i%2){

Serial.println("1 second");

delay(1000);

}

else{

Serial.println("2 seconds");

delay(2000);

}

}

}

Output

Experiment 17

Aim

To blink LEDs for all composite numbers below 15.

Tinkercad Link

https://www.tinkercad.com/things/cpcjoc7UaTt

Code

// C++ code

//

void setup()

{

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void display(int n)

{

digitalWrite(8, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(9, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(10, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(11, n&1?HIGH:LOW);

}

bool isPrime(int n) {

for(int i=2; i<=n/2; i++) {

if(n % i == 0)

return false;

}

return true;

}

void loop(){

for(int i=1; i<=15;i++)

{

if(!isPrime(i)) {

display(i);

delay(2000);

}

}

}

Output

Experiment 18

Aim

To blink LEDs to count even numbers.

Tinkercad Link

https://www.tinkercad.com/things/0F0UdPUgjv8

Code

// C++ code

//

void setup()

{

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

}

void display(int n)

{

digitalWrite(8, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(9, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(10, n&1?HIGH:LOW);

n >>= 1;

digitalWrite(11, n&1?HIGH:LOW);

}

void loop(){

for(int i=1; i<=15;i++)

{

if(i%2 == 0) {

display(i);

delay(2000);

}

}

}

Output

Experiment 19

Aim

To interface an ultrasonic sensor and display the distance measured in a screen and glow red, blue and green led if the distance is close, moderate and far respectively and to play different tones for

different cases.

Tinkercad Link

https://www.tinkercad.com/things/gP7RMdMhAuJ

Code

// C++ code

//

const int redPin = 3;

const int bluePin = 4;

const int greenPin = 5;

const int speakerPin = 10;

const int pingPin = 7;

void setup() {

// initialize serial communication:

Serial.begin(9600);

pinMode(redPin, OUTPUT);

pinMode(bluePin, OUTPUT);

pinMode(greenPin, OUTPUT);

pinMode(speakerPin, OUTPUT);

}

void displayLight(int code) {

digitalWrite(redPin, code == 0 ? HIGH : LOW);

digitalWrite(bluePin, code == 1 ? HIGH : LOW);

digitalWrite(greenPin, code == 2 ? HIGH : LOW);

}

void playTone(long duration, int freq) {

duration *= 1000;

int period = (1.0 / freq) * 100000;

long elapsed_time = 0;

while (elapsed_time < duration) {

digitalWrite(speakerPin, HIGH);

delayMicroseconds(period / 2);

digitalWrite(speakerPin, LOW);

delayMicroseconds(period / 2);

elapsed_time += (period);

}

}

void loop() {

long duration, inches, cm;

pinMode(pingPin, OUTPUT);

digitalWrite(pingPin, LOW);

delayMicroseconds(2);

digitalWrite(pingPin, HIGH);

delayMicroseconds(5);

digitalWrite(pingPin, LOW);

pinMode(pingPin, INPUT);

duration = pulseIn(pingPin, HIGH);

inches = microsecondsToInches(duration);

cm = microsecondsToCentimeters(duration);

Serial.print(inches);

Serial.print("in, ");

Serial.print(cm);

Serial.print("cm");

Serial.println();

if (cm < 50) {

displayLight(0);

playTone(300, 100);

} else if (cm < 150) {

displayLight(1);

playTone(300, 200);

} else {

displayLight(2);

playTone(300, 300);

}

delay(100);

}

long microsecondsToInches(long microseconds) {

return microseconds / 74 / 2;

}

long microsecondsToCentimeters(long microseconds) {

return microseconds / 29 / 2;

}

Output

Experiment 20

Aim

To interface a PIR sensor with Arduino and blink a led if any movement is captured by the sensor and count the number of movements and make a buzzer when movement detected.

Tinkercad Link

https://www.tinkercad.com/things/0Luze9yHsJT

Code

// C++ code

//

const int pirPin = 3;

const int ledPin = 4;

const int speakerPin = 8;

int totalMovements = 0;

bool isMoving = false;

void setup()

{

pinMode(ledPin, OUTPUT);

pinMode(pirPin, INPUT);

pinMode(speakerPin, OUTPUT);

Serial.begin(9600);

}

void playTone(long duration, int freq) {

duration *= 1000;

int period = (1.0 / freq) * 100000;

long elapsed_time = 0;

while (elapsed_time < duration) {

digitalWrite(speakerPin, HIGH);

delayMicroseconds(period / 2);

digitalWrite(speakerPin, LOW);

delayMicroseconds(period / 2);

elapsed_time += (period);

}

}

void loop()

{

int pirStat = digitalRead(pirPin);

if (pirStat == HIGH) {

if (!isMoving) {

playTone(300, 300);

totalMovements += 1;

Serial.print("total movements: ");

Serial.print(totalMovements);

Serial.print("\n");

isMoving = true;

}

digitalWrite(ledPin, HIGH);

} else {

isMoving = false;

digitalWrite(ledPin, LOW);

}

}

Output

Experiment 21

1.

Question

Interface two 7-segment display with Arduino Uno board. Implement the a)odd counter and b)even counter and display the values in the seven-segment display.

Aim

To interface two 7-segment display with Arduino Uno board and to implement odd counter and even counter and display the values in the seven-segment display.

Tinkercad Link

https://www.tinkercad.com/things/jg8jYolPxuL

Code

Serial Monitor

Output

2.

Question

Interface the ambient light sensor with Arduino Uno board. Check the light value from the sensor and switch on/off the bulb (based on the threshold value).
Aim

To interface the ambient light sensor with Arduino Uno board and to  check the light value from the sensor and switch on/off the bulb (based on the threshold value).

Tinkercad Link

https://www.tinkercad.com/things/8sHMma0csEW

Code

Serial Monitor

Output

3.

Question

Interface the temperature and gas sensor with Arduino Uno board. Check the temperature and the gas value, if the limit is beyond the threshold, switch on the bulb and make alarm using buzzer.

Aim

To interface the temperature and gas sensor with Arduino Uno board and check the temperature and the gas value, if the limit is beyond the threshold, switch on the bulb and make alarm using buzzer.

Tinkercad Link

https://www.tinkercad.com/things/dP0QlrHk0YX

Code

Serial Monitor

Output

106119047 | Hari Rahul V | Embedded Systems Lab Report 9

1.

Question

Interface force sensor with Arduino board and display the amount of force given as input to sensor in LCD screen. Also, interface LED RGB and change color of the LED based on some threshold values.

Aim

To interface force sensor with Arduino board and display the amount of force given as input to sensor in LCD screen. To interface LED RGB and change color of the LED based on some threshold values.

Tinkercad Link

https://www.tinkercad.com/things/5t3uKnviROj

Code

#include <LiquidCrystal.h>

const int rs = 12, en = 11, d4 = 5, d5 = 4, d6 = 3, d7 = 2;

LiquidCrystal lcd(rs, en, d4, d5, d6, d7);

const int redPin = 8;

const int bluePin = 9;

const int greenPin = 10;

const int forcePin = A0;

const int blueLimit = 125;

const int redLimit = 350;

void blinkLED(int pinNumber) {

digitalWrite(bluePin, LOW);

digitalWrite(redPin, LOW);

digitalWrite(greenPin, LOW);

digitalWrite(pinNumber, HIGH);

}

void setup() {

pinMode(redPin, OUTPUT);

pinMode(greenPin, OUTPUT);

pinMode(bluePin, OUTPUT);

pinMode(forcePin, INPUT);

lcd.begin(16, 2);

Serial.begin(9600);

}

void loop() {

int force = analogRead(forcePin);

Serial.println(force);

if (force > redLimit) {

blinkLED(redPin);

} else if (force > blueLimit) {

blinkLED(bluePin);

} else {

blinkLED(greenPin);

}

lcd.clear();

lcd.print("Force: ");

lcd.print(force);

delay(100);

}

Serial Monitor

Output

i) Low Force

ii) Medium Force

iii) High Force

2.

Question

Interface the keypad and tilt sensor with Arduino board, if the sensor is being tilted, then take input from keypad and print it in the LCD.

Aim

To interface the keypad and tilt sensor with Arduino board, if the sensor is being tilted, then take input from keypad and print it in the LCD.

Tinkercad Link

https://www.tinkercad.com/things/5MIIrTOqS2g

Code

#include <Keypad.h>

#include <LiquidCrystal.h>

const int rs = 12, en = 11, d4 = 5, d5 = 4, d6 = 3, d7 = 2;

LiquidCrystal lcd(rs, en, d4, d5, d6, d7);

const int numRows = 4;

const int numColumns = 4;

const int tiltPin = 10;

char keys[numRows][numColumns] = {

{'1','2','3', 'A'},

{'4','5','6', 'B'},

{'7','8','9', 'C'},

{'*','0','#', 'D'}

};

byte rowPins[numRows] = {9, 8, A5, A4};

byte columnPins[numColumns] = {A3, A2, A1, A0};

Keypad keypad = Keypad( makeKeymap(keys), rowPins, columnPins, numRows, numColumns );

void setup(){

Serial.begin(9600);

lcd.begin(16, 2);

pinMode(tiltPin, INPUT);

}

void loop(){

char key = keypad.getKey();

if(key) {

Serial.println("Key Pressed");

}

int tiltValue = digitalRead(tiltPin);

if (key && tiltValue == 0){

lcd.setCursor(0, 0);

lcd.print(key);

Serial.println(key);

}

if (tiltValue == 1) {

Serial.println("No Tilt");

} else {

Serial.println("Tilted");

}

delay(100);

}

Output

i)Key not pressed / No Tilt

ii) Key Pressed / No Tilt

iii) Key Not Pressed / Tilt

iv) Key Pressed / Tilt